PROJECT SYNOPSIS

Post-Quantum Cryptography Protected LoRa IoT Gateway

Communication System with Edge AI Integration

Prepared by: Abdul Rahman

Final Year B.Tech Project

Integral University, Lucknow

Academic Year: 2025–2026

# 1. Title of the Project

Title: Post-Quantum Cryptography Protected LoRa IoT Gateway Communication System with Edge AI Integration

# 2. Introduction

## Background

The rapid proliferation of Internet of Things (IoT) devices — estimated at over 16 billion globally — has created an expansive attack surface for adversaries. Conventional IoT security protocols rely on classical asymmetric cryptography such as RSA and Elliptic Curve Diffie-Hellman (ECDH), which are mathematically vulnerable to Shor's algorithm running on a sufficiently powerful quantum computer. The U.S. National Institute of Standards and Technology (NIST) finalized its first three Post-Quantum Cryptography (PQC) standards in August 2024: FIPS 203 (ML-KEM, derived from CRYSTALS-Kyber), FIPS 204 (ML-DSA, derived from CRYSTALS-Dilithium), and FIPS 205 (SLH-DSA, derived from SPHINCS+). Simultaneously, Low-Power Wide-Area Network (LPWAN) technologies — particularly LoRa (Long Range) radio — have emerged as the dominant physical layer for IoT deployments requiring multi-kilometre range without cellular or internet infrastructure. LoRa operates at 433/868/915 MHz ISM bands with configurable air data rates from 0.3 to 19.2 kbps, making it ideal for remote, disaster-struck, or infrastructure-poor environments. However, the intersection of NIST-standard PQC and LoRa-based IoT communication represents a critical and largely unaddressed engineering challenge: PQC key and ciphertext sizes (ML-KEM-768: 1,184-byte public key, 1,088-byte ciphertext; Falcon-512: 666-byte signature) impose severe constraints on LoRa's limited packet capacity (51–255 bytes per transmission), necessitating novel packet fragmentation, session management, and bandwidth optimization strategies that have not been studied in the existing literature.

## Motivation

Current LoRa IoT security implementations rely exclusively on classical symmetric encryption (AES-128/256-CBC) for payload protection, with no quantum-resistant key exchange or authentication. Existing work (Figlarz & Hessel, 2024; Storozhko et al., 2025) has begun exploring PQC in LoRa contexts but is limited to simulation environments, code-based algorithms not standardized by NIST, or LoRaWAN protocol-layer substitutions without gateway-level architectural integration. No existing system combines: (1) NIST-standard ML-KEM and Falcon-512 for live ephemeral key exchange and authentication; (2) LoRa as the sole communication medium; (3) an offline Large Language Model (LLM) accessible via LoRa relay; and (4) multi-modal communication (text, voice, AI query, voice-to-AI) in a unified, internet-free architecture. This project addresses all four simultaneously, filling a clear research gap documented across 13 reviewed papers.

# 3. Problem Statement

Classical asymmetric cryptography (RSA, ECDH, ECDSA) deployed in current LoRa IoT gateway communication systems is mathematically broken by Shor's algorithm on quantum computers. As distributed quantum computing matures (Jiang et al., 2023 demonstrate RSA-2048 decomposition with ~1,047 qubits per machine), the "harvest now, decrypt later" threat model makes migration to quantum-resistant cryptography urgent — particularly for IoT devices with long operational lifespans. However, the direct integration of NIST-standardized PQC algorithms into LoRa IoT gateway systems presents three unsolved engineering problems:

- Bandwidth–Security Mismatch: ML-KEM-768 requires transmitting 3,938 bytes of handshake material (public key + ciphertext + Falcon signature) over a 2.4 kbps LoRa channel, creating a 6–8 second one-time session establishment overhead that must be carefully managed at the protocol level.

- No Existing PQC+LoRa+EdgeAI Integration: No published system relays LLM queries and responses (requiring token-limited, chunked transmission) over a PQC-secured LoRa radio link in an air-gapped, internet-free deployment.

- Lack of Multi-Modal PQC Communication Framework: Existing PQC-on-LoRa proposals address only data payload encryption, not a unified framework supporting text messaging, compressed voice (Codec2), AI text queries, and speech-to-AI workflows over a single secure channel.

# 4. Objectives

## Primary Objectives

I.  To design and implement a Post-Quantum Cryptography session layer for LoRa IoT gateway communication using NIST-standardized ML-KEM (Kyber-768) for ephemeral key encapsulation and Falcon-512 for mutual authentication, achieving IND-CCA2 secure, forward-secret, replay-resistant communication over a 2.4 kbps LoRa radio link.

II.  To demonstrate a four-mode offline communication system (person-to-person text, person-to-AI text, voice message via Codec2, and voice-to-AI via on-device STT) operating exclusively over PQC-secured LoRa radio without internet, Wi-Fi, GSM, or Bluetooth dependency.

## Secondary Objectives

I.  To evaluate and benchmark the performance of the system across key metrics including PQC handshake latency, per-mode message latency, packet delivery ratio, communication range (up to 10 km), and power consumption of the ESP32 S3 + EBYTE E32 gateway node.

II.  To integrate an offline Large Language Model (Ollama) as an Edge AI layer accessible via LoRa-relayed queries, with response chunking and token-limiting optimized for LoRa's bandwidth constraints, demonstrating the feasibility of AI-assisted communication in infrastructure-denied environments.

# 5. Scope of the Project

## Included in Scope

- Hardware assembly and firmware development for three ESP32 S3 + EBYTE E32-433T30D-V8 LoRa gateway nodes

- Android application (Kotlin) with Bouncy Castle PQC library for ML-KEM-768 and Falcon-512 operations, USB CDC communication, Codec2 audio encoding/decoding, and on-device STT

- PC Python server (FastAPI + liboqs) for PQC session management, LoRa packet relay, Ollama LLM integration, and audio reassembly

- PQC session handshake: ML-KEM-768 key encapsulation + Falcon-512 mutual authentication + HKDF-derived AES-256-GCM session key

- All four communication modes: Mode 1 (text), Mode 2 (AI text), Mode 3 (voice via Codec2), Mode 4 (voice-to-AI via Android STT)

- Performance evaluation: range, latency per mode, packet delivery ratio, handshake timing, power consumption, and security demonstration

- IEEE conference paper submission based on evaluation results

## Excluded from Scope

- Wi-Fi, Bluetooth, GSM, cellular, or internet-based communication paths

- LoRaWAN network server / cloud infrastructure (pure LoRa P2P only)

- Side-channel attack resistance analysis or hardware security module (HSM) integration

- Real-time voice streaming (system uses store-and-forward Codec2 audio, not live audio)

- Post-quantum TLS or HTTPS — the system uses a custom PQC session layer, not TLS

- Custom ASIC/FPGA hardware acceleration for PQC operations

# 6. Methodology

The project follows an iterative hardware-software co-design methodology structured in six sequential phases. The cryptographic architecture is designed with a clear separation of concerns: the ESP32 S3 acts exclusively as a transparent USB-CDC to LoRa-UART bridge with no cryptographic role, while all PQC operations (ML-KEM key generation, Falcon signing/verification, AES-256-GCM encryption/decryption) execute on Android endpoints (via Bouncy Castle) and the PC server (via liboqs). This design ensures that private keys never traverse the radio link and that a physical ESP32 compromise yields no cryptographic material.

Session establishment follows a five-step ephemeral handshake: (1) Initiator generates fresh ML-KEM-768 and Falcon-512 keypairs; (2) Signs the ML-KEM public key with Falcon and transmits both over LoRa; (3) Responder verifies the Falcon signature, runs ML-KEM encapsulation to produce a shared secret, signs the ciphertext with its own Falcon key, and returns it; (4) Initiator verifies, decapsulates, and both parties derive an AES-256-GCM session key via HKDF; (5) All subsequent packets are AES-256-GCM encrypted with unique per-packet nonces derived from the session key and sequence counter. The entire handshake cost is incurred once per session (~6–8 seconds at 2.4 kbps), with zero cryptographic overhead on subsequent messages.

The methodology is validated through controlled experiments measuring all performance metrics defined in Section 11, with results forming the empirical basis of the IEEE conference paper.

# 7. System Design

## a. Architecture

The system architecture comprises three logical layers:

- Physical Radio Layer: Two EBYTE E32-433T30D-V8 LoRa modules operating at 433 MHz with 1W output power, configured at 2.4 kbps (maximum range mode) or 9.6 kbps (indoor mode). ESP32 S3 acts as a transparent UART↔USB-CDC bridge only.

- Cryptographic Session Layer: ML-KEM-768 + Falcon-512 handshake for session establishment; AES-256-GCM with HMAC-SHA256 for all data packets. Runs on Android (Bouncy Castle) and PC (liboqs). ESP32 is cryptographically transparent.

- Application Layer: Android Kotlin app (chat UI, push-to-talk, AI query input, STT) ↔ LoRa link ↔ PC Python FastAPI server (Ollama integration, packet reassembly, session management).

Physical connection chain: Android A → USB-C → ESP32 S3 (A) → UART → EBYTE E32 (A) ←→ 433 MHz LoRa ←→ EBYTE E32 (B) → UART → ESP32 S3 (B) → USB-C → Android B / PC

## b. Components

| Component | Role | Qty |
| --- | --- | --- |
| ESP32 S3 WROOM-1 N16R8 (16MB Flash, 8MB PSRAM) | LoRa-USB bridge, 240 MHz Xtensa LX7, native USB OTG | 3 |
| EBYTE E32-433T30D-V8 LoRa Module | 1W 433 MHz LoRa, UART AT commands, built-in FEC, 0.3–19.2 kbps | 3 |
| TX433-XPA100 Antenna | RF antenna for 433 MHz LoRa module | 3 |
| USB Type-C to Type-C Cable | Android/PC ↔ ESP32 S3 CDC bridge | 3 |
| Android Smartphones (×2) | User endpoints: chat, voice, AI query, STT | 2 |
| Personal Computer (×1) | Ollama LLM server + PC Python FastAPI server | 1 |

# 8. Development Process

## a. Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Android App | Kotlin, Android Studio, Bouncy Castle PQC | UI, PQC crypto, USB CDC, Codec2 JNI, STT |
| PQC (Android) | ML-KEM-768, Falcon-512 (Bouncy Castle) | Key exchange, digital signatures |
| PQC (PC) | liboqs-python (oqs.KeyEncapsulation, oqs.Signature) | PC-side PQC operations |
| Symmetric Crypto | AES-256-GCM (javax.crypto / pycryptodome) | All packet encryption after handshake |
| Audio Codec | Codec2 (C library, JNI wrapper for Android) | Voice compression at 700–1200 bps |
| Speech-to-Text | Android SpeechRecognizer API (offline mode) | Voice-to-AI Mode 4 |
| PC Server | Python 3.11, FastAPI, pyserial | LoRa relay, Ollama integration, packet management |
| Edge AI | Ollama (LLaMA 3, localhost:11434) | Offline LLM inference, 300-token cap |
| ESP32 Firmware | ESP-IDF v5.x, TinyUSB CDC, EBYTE UART driver | Transparent byte bridge, CRC16 check |
| Key Derivation | HKDF-SHA256 | Session key derivation from ML-KEM shared secret |

## b. AI/ML Integration

The system integrates an offline Large Language Model via Ollama running on the local PC (no internet required). Users on Android can send text queries (Mode 2) or voice queries converted via on-device STT (Mode 4) over the PQC-secured LoRa link. The PC server decrypts the query, forwards it to Ollama (localhost:11434), receives the LLM response capped at 300 tokens to fit LoRa bandwidth constraints, chunks the response into ≤200-byte AES-256-GCM encrypted packets (type 0x03), and relays them back over LoRa to the Android device for reassembly and display. This represents the first documented instance of offline LLM inference being made accessible over a post-quantum secured LoRa radio link.

# 9. Implementation Steps

## Phase 1: Hardware Setup & LoRa Bridge (Weeks 1–4)

Assemble all three ESP32 S3 + EBYTE E32 + antenna gateway nodes. Flash TinyUSB CDC firmware using ESP-IDF. Verify USB-C detection from Android (UsbManager) and PC (pyserial). Implement packet structure (type byte, sequence number, total chunks, CRC16, GCM nonce, encrypted payload, GCM auth tag). Test raw byte pass-through. Calibrate LoRa air data rates (2.4 kbps and 9.6 kbps). Conduct initial range and packet loss tests.

## Phase 2: PQC Session Layer (Weeks 5–7)

Implement ML-KEM-768 key generation, encapsulation, and decapsulation on Android using Bouncy Castle (org.bouncycastle:bcprov-jdk15on). Implement Falcon-512 key generation and signing on Android. Mirror all PQC operations on PC using liboqs-python. Implement the five-step session handshake over the live LoRa link. Derive AES-256-GCM session key via HKDF. Verify forward secrecy by discarding sk_A post-handshake. Measure and record handshake timing.

## Phase 3: Application Modes & Evaluation (Weeks 8–14)

Build Android Material Design 3 chat UI and AI query interface. Implement Mode 1 (text chat) and Mode 2 (AI text query via Ollama). Integrate Codec2 JNI library for Mode 3 (voice message: record → encode → chunk → encrypt → transmit → reassemble → decode → play). Integrate Android SpeechRecognizer for Mode 4 (voice-to-AI). Build PC FastAPI server with full Ollama integration and packet reassembly. Conduct full performance evaluation across all metrics (Section 11). Write and submit IEEE conference paper.

# 10. Testing Methodology

| Test Type | What is Tested | Method |
| --- | --- | --- |
| Unit Testing | ML-KEM, Falcon, AES-256-GCM operations | Known-answer tests against NIST FIPS 203/204 test vectors |
| Integration Testing | Full PQC handshake over live LoRa link | End-to-end session establishment between Android A and PC |
| Functional Testing | All four communication modes | Manual testing of text, AI, voice, voice-to-AI across 50m, 500m, 1km, 5km |
| Performance Testing | Latency, range, packet delivery ratio | Timestamped measurements averaged over 20–50 trials per distance |
| Security Testing | Passive eavesdropping, replay attack resistance | Third ESP32 sniffing LoRa channel — verify no plaintext readable |
| Stress Testing | Sustained communication, long messages, audio reassembly | Extended multi-packet message sessions with loss simulation |
| Power Testing | ESP32 + EBYTE current draw | Multimeter measurement during TX, RX, and idle states |

# 11. Expected Outcomes

- A fully functional, demonstrable three-node LoRa IoT gateway system with NIST-standard PQC (ML-KEM-768 + Falcon-512) session security.

- PQC handshake completion in 6–8 seconds over a 2.4 kbps LoRa link — demonstrating feasibility of quantum-resistant key exchange on an extremely bandwidth-constrained radio channel.

- Mode 1 (text) latency under 1 second; Mode 2 (AI text) end-to-end response in 8–15 seconds; Mode 3 (10s voice message) transfer in ~5 seconds; Mode 4 (voice-to-AI) in 9–17 seconds.

- Demonstrated communication range of 1–5 km with packet delivery ratio >95% at 2.4 kbps under line-of-sight conditions.

- Verified security: a passive LoRa sniffer (third ESP32) cannot read any plaintext from the channel.

- Published IEEE conference paper documenting the system design, implementation, and evaluation results.

# 12. Deliverables

- Three assembled ESP32 S3 + EBYTE E32 + antenna LoRa gateway nodes with flashed firmware

- Android application (Kotlin APK) with full PQC stack, four communication modes, Codec2 audio, and STT

- PC Python server (FastAPI + liboqs + Ollama integration) with complete session management and packet handling

- Performance evaluation dataset: latency, range, PDR, handshake time, power consumption measurements

- IEEE conference paper: "Design and Implementation of a Post-Quantum Cryptography Protected LoRa IoT Gateway Communication System with Integrated Edge AI"

- Project report and source code repository

# 13. Impact

This project addresses a critical gap in IoT security infrastructure that will only grow more urgent as quantum computing matures. By demonstrating a working, standards-compliant PQC implementation on a LoRa-based IoT gateway, the project provides:

- A practical blueprint for migrating existing LoRa IoT deployments to quantum-resistant security before cryptographically relevant quantum computers emerge.

- Emergency communication capability: the internet-free, infrastructure-independent architecture is directly applicable to disaster response, military field operations, remote healthcare (rural India), and areas without cellular coverage — addressing SDGs 3, 9, 11, 13, and 16.

- An open research contribution: the first documented performance characterization of ML-KEM + Falcon-512 handshake latency specifically over a 2.4 kbps LoRa radio link.

- A novel Edge AI access paradigm: enabling offline LLM inference to be queried from field devices with no internet, secured by NIST-standard PQC.

# 14. Timeline (Gantt Chart)

| Phase | Duration | W1 | W2 | W3 | W4 | W5 | W6 | W7 | W8 | W9 | W10 | W11 | W12 | W13 | W14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase 1: Hardware Setup | Week 1–2 | ✓ |  |  |  |  |  |  |  |  |  |  |
| Phase 2: LoRa Communication | Week 3–4 |  | ✓ | ✓ |  |  |  |  |  |  |  |  |
| Phase 3: PQC Handshake | Week 5–7 |  |  |  | ✓ | ✓ | ✓ |  |  |  |  |  |
| Phase 4: Mode 1 & 2 (Text+AI) | Week 8–10 |  |  |  |  |  |  | ✓ | ✓ | ✓ |  |  |
| Phase 5: Mode 3 & 4 (Voice) | Week 11–12 |  |  |  |  |  |  |  |  |  | ✓ | ✓ |  |
| Phase 6: Evaluation & Report | Week 13–14 |  |  |  |  |  |  |  |  |  |  |  | ✓ |

# 15. Project Plan

| Phase | Key Tasks | Milestones | Duration |
| --- | --- | --- | --- |
| Phase 1 Hardware & LoRa | Assemble nodes, flash firmware, verify USB-C bridge, test raw LoRa pass-through, measure range | Working LoRa bridge; raw byte pass-through confirmed | Week 1–4 |
| Phase 2 PQC Handshake | Implement ML-KEM-768 + Falcon-512 on Android & PC, run handshake over live LoRa, derive AES-256-GCM key | Shared session key established over LoRa; forward secrecy verified | Week 5–7 |
| Phase 3 Mode 1 & 2 | Android chat UI, PC FastAPI + Ollama, test encrypted text and AI queries end-to-end | Text chat and AI queries working over PQC-secured LoRa | Week 8–10 |
| Phase 4 Mode 3 & 4 | Codec2 JNI, audio packetisation/reassembly, Android STT for voice-to-AI | Voice and voice-to-AI working end-to-end | Week 11–12 |
| Phase 5 Evaluation | Measure all metrics, security demonstration, power testing | Full evaluation dataset; system demo ready | Week 13 |
| Phase 6 Paper & Report | Write IEEE paper, project report, prepare submission | IEEE paper submitted; project report complete | Week 14 |

# 16. Resources Required

## 17. Hardware

| Component | Unit Price | Qty | Total |
| --- | --- | --- | --- |
| ESP32 S3 WROOM-1 N16R8 (16MB Flash, 8MB PSRAM) | ₹799 | 3 | ₹2,397 |
| EBYTE E32-433T30D-V8 LoRa Module (1W, 433 MHz) | ₹1,220 | 3 | ₹3,660 |
| TX433-XPA100 Antenna (433 MHz) | ₹438 | 3 | ₹1,314 |
| USB Type-C to Type-C Cable | ₹150 | 3 | ₹450 |
| Android Smartphones (×2) | Existing | 2 | — |
| Personal Computer (×1) | Existing | 1 | — |
| Total Hardware Cost |  |  | ₹7,821 |

## 18. Software

- Android Studio (free) — Android app development

- ESP-IDF v5.x (free, open-source) — ESP32 S3 firmware

- Python 3.11+ (free) — PC server

- Bouncy Castle PQC (free, open-source) — ML-KEM-768 + Falcon-512 on Android

- liboqs-python (free, open-source) — PQC operations on PC

- Ollama (free, open-source) — Local LLM inference

- Codec2 (free, open-source, LGPL) — Audio codec

- FastAPI (free, open-source) — PC Python server framework

- pycryptodome (free, open-source) — AES-256-GCM on PC

# 19. References

## 20. Literature

[1] A. Shaw, "quantum-safe: Bridging the Post-Quantum Production Gap with a Hybrid-by-Default Python Cryptography Library," IEEE Transactions on Information Forensics and Security, May 2026.

[2] J.-R. Jiang, T.-Y. Wang, W.-H. Huang, J.-Z. Zhang, "Distributed Shor's Algorithm with Sequential Quantum Teleportation," IEEE ECICE, 2023.

[3] G. R. Figlarz, F. P. Hessel, "Enhancement in LoRaWAN's Security With Post-Quantum Key Encapsulation Method," IEEE WF-IoT, 2024.

[4] A. Storozhko et al., "Enhancing LoRa Encryption with Post-Quantum Algorithms for IoT Security," IEEE KhPIWeek, 2025.

[5] P.-A. Fouque et al., "Falcon: Fast-Fourier Lattice-based Compact Signatures over NTRU — Specification v1.2," NIST PQC Submission, 2020.

[6] S. I. Soto-Ortiz et al., "LP Gas Leak Detection Prototype for Rural Areas with AES-256 Encryption via LoRa and 3G Network Transmission," IEEE ENC, 2025.

[7] NIST, "FIPS 203: Module-Lattice-Based Key-Encapsulation Mechanism Standard," August 2024. DOI: 10.6028/NIST.FIPS.203.

[8] NIST, "FIPS 204: Module-Lattice-Based Digital Signature Standard," August 2024. DOI: 10.6028/NIST.FIPS.204.

[9] P. Hidayatulloh et al., "Performance Evaluation of ESP32 Random Number Generator for LoRa Communication Security," IEEE ICRAMET, 2024.

[10] P. Govindhan, K. Kumar, "Post-Quantum Cryptography for Multiple High-Resolution Millimeter Wave Images for Enhanced Security in IoT Networks," IEEE InCACCT, 2024.

[11] N. Puch, M. Pursche, S. N. Peters, M. P. Heinl, "SoK: The Engineer's Guide to Post-Quantum Cryptography for Embedded Devices," IEEE EuroS&P, 2026.

[12] H. Yanagimoto, I. Kisaku, K. Hashimoto, "Table-To-Text Using Pre-trained Large Language Model and LoRA," IEEE IIAI-AAI, 2024.

## 21. Tools and Technologies

- NIST PQC Standards: FIPS 203, FIPS 204, FIPS 205 — https://csrc.nist.gov/pqcrypto

- Bouncy Castle PQC — https://www.bouncycastle.org/java.html

- liboqs (Open Quantum Safe) — https://github.com/open-quantum-safe/liboqs

- Ollama — https://ollama.com

- Codec2 — https://github.com/drowe67/codec2

- ESP-IDF — https://docs.espressif.com/projects/esp-idf

- EBYTE E32 LoRa Module — https://www.ebyte.com

# 22. Appendices

## 23. Additional Information

### A. PQC Packet Structure

| Field | Size | Description |
| --- | --- | --- |
| Type Byte | 1 byte | 0x01=text, 0x02=AI query, 0x03=AI response, 0x04=audio, 0x05=handshake |
| Sequence Number | 1 byte | 0–255 wrapping; used for audio reassembly and duplicate detection |
| Total Chunks | 1 byte | Number of chunks this message spans (1 for single-packet messages) |
| CRC16 | 2 bytes | Over entire packet; ESP32 verifies before forwarding |
| GCM Nonce | 12 bytes | Unique per packet; derived from session key + sequence counter |
| Encrypted Payload | ≤220 bytes | AES-256-GCM ciphertext containing actual message data |
| GCM Auth Tag | 16 bytes | GCM integrity tag; receiver verifies before decrypting |

### B. Session Handshake Steps

| Step | Actor | Action |
| --- | --- | --- |
| 1 | Android A / PC | Generate fresh ML-KEM-768 keypair (pk_A, sk_A) and Falcon-512 keypair (fpk_A, fsk_A) |
| 2 | Android A | Sign pk_A with fsk_A → sig_A. Transmit (pk_A, fpk_A, sig_A) over LoRa |
| 3 | Android B / PC | Receive and verify sig_A using fpk_A. Abort if invalid. |
| 4 | Android B / PC | Run ML-KEM Encaps(pk_A) → (ciphertext_K, shared_secret). Sign ciphertext_K → sig_B. Send back. |
| 5 | Android A | Verify sig_B. Run ML-KEM Decaps(sk_A, ciphertext_K) → shared_secret. Discard sk_A. |
| 6 | Both | Derive AES-256-GCM key = HKDF(shared_secret, "session-key"). Session live. |

### C. Communication Mode Latency Summary

| Mode | Operation | Estimated Latency |
| --- | --- | --- |
| Session Start (all modes) | ML-KEM + Falcon handshake over LoRa at 2.4 kbps | 6–8 seconds (one-time only) |
| Mode 1 — Text Chat | Short message (~100 bytes) end-to-end | < 1 second |
| Mode 2 — AI Text Query | Query + Ollama inference + chunked response over LoRa | 8–15 seconds |
| Mode 3 — Voice Message | 10 seconds of Codec2 audio encoded and transferred | ~5 seconds transfer |
| Mode 4 — Voice to AI | On-device STT + same as Mode 2 | 9–17 seconds total |