Post-Quantum Cryptography Protected

LoRa IoT Gateway Communication System with Edge AI Integration

Full Technical Project Plan

LoRa · ESP32 S3 · ML-KEM · Falcon-512 · AES-256-GCM · Ollama · Codec2

Prepared by:

Abdul Rahman

# 1. Project Overview

This document defines the complete technical architecture for a Post-Quantum Cryptography Protected LoRa IoT Gateway Communication System with Edge AI Integration. The system enables four distinct communication modes over a LoRa radio mesh — without any reliance on Wi-Fi, Bluetooth, GSM, or the internet — protected against the threat of quantum computers using real Post-Quantum Cryptography (PQC) as standardised by NIST.

## 1.1  Project Title

Post-Quantum Cryptography Protected LoRa IoT Gateway Communication System with Edge AI Integration

## 1.2  Core Objective

To build a working, demonstrable system in which:

- Android smartphones communicate with each other over distances of 1–10 km using LoRa 433 MHz radio.

- Every message — text, audio, or AI query — is encrypted using AES-256-GCM, with session keys derived via Kyber-768 (ML-KEM), the NIST-standardised post-quantum key encapsulation algorithm.

- Device identities are authenticated each session using Falcon-512, the NIST-standardised post-quantum digital signature algorithm.

- An offline Large Language Model (Ollama) hosted on a PC can answers queries forwarded over the LoRa network, including speech-converted queries from Android.

- No proprietary cloud, no internet, no GSM.

## 1.3  What Is Included

| Technology | Decision and Reason |
| --- | --- |
| ML-KEM (Kyber-768) | INCLUDED — NIST standard PQC key exchange. Live ephemeral handshake every session. |
| Falcon-512 | INCLUDED — NIST standard PQC digital signature. Used once per session for auth. |
| AES-256-GCM | INCLUDED — Symmetric encryption for every packet after key exchange. |
| HMAC-SHA256 | INCLUDED — Per-chunk integrity on audio reassembly. |
| Codec2 (700–1200 bps) | INCLUDED — Only audio codec that fits LoRa bandwidth. |
| Android STT API | INCLUDED — On-device speech-to-text for Mode 4. No audio over LoRa. |
| Ollama (local LLM) | INCLUDED — Offline AI on PC, no internet required. |

# 2. Hardware

## 2.1  Component List (3 Gateway Devices)

| Component | Role | Unit Price | Qty | Total |
| --- | --- | --- | --- | --- |
| ESP32 S3 WROOM-1 N16R8 (16MB Flash, 8MB PSRAM) | Gateway controller, USB CDC bridge | ₹799 | 3 | ₹2,397 |
| EBYTE E32-433T30D-V8 LoRa Module | Long-range radio (1W, 433 MHz) | ₹1,220 | 3 | ₹3,660 |
| TX433-XPA100 Antenna (433 MHz) | RF antenna for LoRa module | ₹438 | 3 | ₹1,314 |
| USB Type-C to Type-C Cable | Android/PC ↔ ESP32 bridge | ₹150 | 3 | ₹450 |
| Android Smartphones (×2) | User devices (existing) | — | 2 | — |
| Personal Computer (×1) | Ollama AI server (existing) | — | 1 | — |

## 2.2  Why ESP32 S3 Specifically

- Native USB OTG — supports USB CDC class without external USB-serial chip. Android sees it as a standard serial device via UsbManager API.

- 8 MB PSRAM — sufficient RAM for packet buffering and LoRa UART bridge without running out of heap.

- 240 MHz dual-core Xtensa LX7 — fast enough for CRC16 checks and packet routing at LoRa speeds.

- ESP32 S3 does NOT need to run any cryptography. It is a transparent bridge only.

## 2.3  Why EBYTE E32-433T30D-V8

- UART interface — simple AT command control, no SPI timing issues.

- 1 W output power — maximum legal power for 433 MHz ISM band in India (865–867 MHz band is also available but this module is optimised for 433 MHz).

- Air data rates 0.3–19.2 kbps — configurable. Set to 2.4 kbps for maximum range, 9.6 kbps for faster transfer indoors.

- Built-in forward error correction (FEC) — reduces packet loss over long distances.

## 2.4  Physical Connection Diagram

Android A USB-C → ESP32 S3 (A) → UART → EBYTE E32 (A) ←→ 433 MHz LoRa ←→ EBYTE E32 (B) → UART → ESP32 S3 (B) → USB-C → Android B / PC

The ESP32 bridges USB CDC (Android/PC side) to UART (LoRa module side). It reads bytes from USB and writes them to the LoRa UART, and vice versa. It has no knowledge of the content — all bytes are encrypted before they reach the ESP32.

# 3. Cryptography — Post-Quantum Security

## 3.1  Why Static Hardcoded Keys Are Not Acceptable

A common mistake is to generate key pairs once, hardcode them into firmware, and call this "post-quantum encryption." This approach fails on three grounds:

- It is not Kyber/ML-KEM. Kyber is defined by its ephemeral key generation — a new keypair is generated fresh every session. Hardcoded keys are simply classical asymmetric crypto with a fixed key, equivalent to RSA with the private key stored in flash.

- Forward secrecy is broken. If an attacker dumps the ESP32 flash (trivial with a programmer), they can decrypt all past and future messages.

- No examiner or reviewer will accept "hardcoded asymmetric keys" as an implementation of ML-KEM. The NIST standard specifically requires ephemeral operation.

Correct design: Kyber operations run on Android (Bouncy Castle PQC library) and PC (liboqs). The ESP32 carries encrypted bytes it cannot read. Private keys never leave the device that generated them and are discarded after each session.

## 3.2  Session Handshake (Runs Once Per Session)

The following sequence establishes a shared AES-256 session key between two endpoints using Kyber-768 and authenticates both parties with Falcon-512:

| Step | Who Does It | What Happens |
| --- | --- | --- |
| 1 | Android A (or PC) | Generate fresh Kyber-768 keypair (pk_A, sk_A). Generate fresh Falcon-512 keypair (fpk_A, fsk_A). |
| 2 | Android A | Sign pk_A with fsk_A → signature sig_A. Send (pk_A, fpk_A, sig_A) over USB-C → ESP32 → LoRa. |
| 3 | Android B / PC | Receive (pk_A, fpk_A, sig_A). Verify sig_A using fpk_A. If invalid, abort. |
| 4 | Android B / PC | Run Kyber Encapsulate(pk_A) → (ciphertext_K, shared_secret). Sign ciphertext_K with fsk_B → sig_B. Send (ciphertext_K, fpk_B, sig_B) back over LoRa. |
| 5 | Android A | Receive (ciphertext_K, fpk_B, sig_B). Verify sig_B using fpk_B. Run Kyber Decapsulate(sk_A, ciphertext_K) → shared_secret. |
| 6 | Both sides | Derive AES-256-GCM key = HKDF(shared_secret, "session-key"). Discard sk_A. Session is live. |

## 3.3  Handshake Timing Budget

| Operation | Estimated Time |
| --- | --- |
| Kyber-768 keygen on Android (Xtensa LX7 class) | ~5–10 ms |
| Falcon-512 sign on Android | ~10–20 ms |
| Kyber public key over LoRa (1,184 bytes @ 2.4 kbps) | ~3.9 s |
| Kyber ciphertext over LoRa (1,088 bytes @ 2.4 kbps) | ~3.6 s |
| Falcon signature over LoRa (666 bytes @ 2.4 kbps) | ~2.2 s |
| Total one-time handshake cost | ≈ 6–8 seconds |
| Per-message cost after handshake | 0 ms (AES-256-GCM only) |

The handshake happens once when a session opens. All subsequent messages use the derived AES-256-GCM key with negligible overhead.

## 3.4  Per-Packet Encryption (AES-256-GCM)

Every packet across all four communication modes uses the same structure:

| Field | Size | Content |  |  |
| --- | --- | --- | --- | --- |
| Type byte | 1 byte | 0x01=text, 0x02=AI query, 0x03=AI response chunk, 0x04=audio chunk, 0x05=handshake |  |  |
| Sequence number | 1 byte | 0–255, wraps. Used for audio reassembly and duplicate detection. |  |  |
| Total chunks | 1 byte | How many chunks this message spans. 1 for single-packet messages. |  |  |
| CRC16 | 2 bytes | Over the entire packet. ESP32 checks this before forwarding. |  |  |
| GCM nonce | 12 bytes | Unique per packet. Derived from session key + sequence counter. |  |  |
| Encrypted payload | ≤220 bytes | AES-256-GCM ciphertext. Contains actual message data. |  |  |
| GCM auth tag | 16 bytes | GCM integrity tag. Receiver verifies before decrypting. |  |  |

# 4. Four Communication Modes

## Mode 1 — Person-to-Person Text Chat

| Stage | Detail |
| --- | --- |
| 1. User types message | Android A keyboard input. Any length; long messages are split into multiple packets automatically. |
| 2. Encrypt on Android A | AES-256-GCM with session key. Attach packet type 0x01, sequence number, CRC16. |
| 3. Send over USB-C | Android UsbManager bulk transfer to ESP32 S3 CDC endpoint. |
| 4. ESP32 A forwards | CRC16 check. Write bytes to EBYTE E32 UART. LoRa transmission. |
| 5. ESP32 B receives | Read bytes from EBYTE E32 UART. CRC16 check. Write to USB-C CDC. |
| 6. Decrypt on Android B | AES-256-GCM decrypt with session key. Verify GCM tag. Display message. |
| Latency | Under 1 second for a typical short message at 2.4 kbps air rate. |

## Mode 2 — Person-to-AI Text (Ollama on PC)

| Stage | Detail |
| --- | --- |
| 1. User types AI query | Android A. Packet type 0x02. |
| 2. Encrypt and send | AES-256-GCM → USB-C → ESP32 A → LoRa → ESP32 B → USB-C → PC. |
| 3. PC Python server | Decrypts with session key. Sends plaintext to Ollama API (localhost:11434). |
| 4. Ollama generates reply | Local LLM inference. Response capped at 300 tokens to limit LoRa payload size. |
| 5. PC chunks and sends | PC splits response into 200-byte chunks. Each chunk: type 0x03, sequence number, encrypted. Sent back over LoRa. |
| 6. Android A reassembles | Collect all 0x03 chunks by sequence number. Decrypt and concatenate. Display full response. |
| Latency | ~2–5 s Ollama inference + ~5–10 s LoRa transfer = ~8–15 s total. Acceptable for offline AI. |

## Mode 3 — Person-to-Person Voice Message

| Stage | Detail |
| --- | --- |
| 1. Record audio | Android A microphone. PCM 8000 Hz, mono, 16-bit. User holds record button. |
| 2. Codec2 encode | Compress PCM to 700–1200 bps using Codec2 library (ported to Android). Output: raw Codec2 bitstream. |
| 3. Packetise | Split Codec2 bitstream into 20-byte data chunks. Assign sequence numbers. Type 0x04. |
| 4. Encrypt each chunk | AES-256-GCM per chunk. Unique nonce per packet. |
| 5. Transmit over LoRa | ESP32 A → LoRa → ESP32 B. One LoRa packet per audio chunk. |
| 6. Receive and reassemble | Android B collects chunks by sequence number. HMAC-SHA256 validates reassembled bitstream. |
| 7. Codec2 decode | Decode Codec2 bitstream to PCM. Play through Android speaker. |
| Audio quality | Intelligible voice. Walkie-talkie quality. Not music-grade. Transfer is store-and-forward, not real-time. |
| Transfer time (10s audio) | ~10s of Codec2 @ 1200 bps = ~1,500 bytes. At 2.4 kbps LoRa ≈ 5 seconds transfer. |

## Mode 4 — Voice-to-AI (Speech-to-Text on Sender, Text to Ollama)

| Stage | Detail |
| --- | --- |
| 1. Record voice | Android A microphone. PCM input. |
| 2. Speech-to-Text on Android A | Android built-in SpeechRecognizer API (offline mode). Converts voice to text entirely on the sender's phone. No audio ever leaves Android A. |
| 3. Text query produced | Plain text output from STT. This is treated identically to a Mode 2 typed query from this point forward. |
| 4. Encrypt and send text | AES-256-GCM. Type 0x02. USB-C → ESP32 A → LoRa → ESP32 B → USB-C → PC. |
| 5. PC → Ollama → reply | Same as Mode 2. Response chunked and sent back. |
| 6. Display on Android A | Text response displayed. No audio returned. |
| Key design point | Codec2 audio is NOT used in this mode. Only text travels over LoRa. This keeps Mode 4 latency similar to Mode 2 (~8–15 s total). |

# 5. Software Stack

## 5.1  Android Application

| Component | Technology and Notes |
| --- | --- |
| Language | Kotlin (primary), Java interop for Bouncy Castle PQC |
| IDE | Android Studio |
| USB-C bridge | android.hardware.usb.UsbManager — BulkTransfer in/out to ESP32 CDC endpoint |
| ML-KEM (Kyber-768) | Bouncy Castle PQC — org.bouncycastle:bcprov-jdk15on — KeyPairGenerator("KYBER", "BC") |
| Falcon-512 | Bouncy Castle PQC — Signature("FALCON-512", "BC") |
| AES-256-GCM | javax.crypto.Cipher("AES/GCM/NoPadding") — standard Android API |
| Codec2 audio | Codec2 native library (C) — JNI wrapper for Android. Open-source: github.com/drowe67/codec2 |
| Speech-to-Text (Mode 4) | android.speech.SpeechRecognizer — offline recognition mode |
| Local storage | SQLite via Room — message history, session keys (ephemeral, cleared on close) |
| UI | Material Design 3 — chat bubble layout, push-to-talk button, AI query input |

## 5.2  ESP32 S3 Firmware

| Component | Technology and Notes |
| --- | --- |
| Framework | ESP-IDF v5.x (preferred) or Arduino IDE with ESP32 S3 board support |
| USB CDC | TinyUSB CDC class — built into ESP-IDF. Presents as /dev/ttyACM0 on PC, UsbManager device on Android |
| LoRa UART | ESP UART driver — reads/writes bytes to EBYTE E32 module at 9600 baud (configurable) |
| Packet routing | Read bytes from USB CDC, check CRC16, write to LoRa UART. And vice versa. No crypto. |
| No crypto on ESP32 | The ESP32 is a transparent bridge. It never sees plaintext. All crypto runs on Android and PC. |

## 5.3  PC Python Server

| Component | Technology and Notes |
| --- | --- |
| Language | Python 3.11+ |
| Framework | FastAPI — async HTTP server for internal routing and Ollama API calls |
| USB-C serial | pyserial — reads/writes bytes from ESP32 CDC (/dev/ttyACM0 or COMx on Windows) |
| ML-KEM (Kyber-768) | liboqs-python — pip install liboqs — oqs.KeyEncapsulation("Kyber768") |
| Falcon-512 | liboqs-python — oqs.Signature("Falcon-512") |
| AES-256-GCM | pycryptodome — AES.new(key, AES.MODE_GCM) |
| HMAC-SHA256 | Python hmac module — standard library |
| Ollama integration | ollama Python client — ollama.chat(model="llama3", messages=[...]) — localhost:11434 |
| Local storage | SQLite — user registry, message logs, session records |
| Packet reassembly | Python dict keyed by (session_id, chunk_type) — collects chunks, yields complete messages |

# 6. Latency Summary

All timings assume LoRa air data rate of 2.4 kbps (maximum range mode). At 9.6 kbps (shorter range), divide LoRa transfer times by ~4.

| Mode | Operation | Estimated Latency |
| --- | --- | --- |
| Session start (all modes) | ML-KEM + Falcon handshake over LoRa | 6–8 seconds, one time only |
| Mode 1 — text chat | Short message (~100 bytes) end-to-end | < 1 second |
| Mode 2 — AI text query | Query + Ollama inference + response transfer | 8–15 seconds |
| Mode 3 — voice message | 10 seconds of Codec2 audio transfer | ~5 seconds transfer time |
| Mode 4 — voice to AI | STT on Android + same as Mode 2 | 9–17 seconds (STT ~1–2 s added) |

# 7. Novelty of This System

Each component used in this project exists independently in the literature. The novelty lies in their integration:

- LoRa communication systems exist, but none combine them with NIST-standard post-quantum cryptography at the session layer.

- ML-KEM and Falcon-512 are NIST-standardised (2024) but have not been widely deployed on LoRa IoT gateway systems.

- Offline LLM integration (Ollama) via LoRa radio — queries and responses relayed over a low-bandwidth radio link — is novel.

- The four-mode unified platform (text, voice message, AI text, voice-to-AI) over a single PQC-secured LoRa channel is a distinctive engineering contribution.

- The architecture correctly separates concerns: ESP32 as transparent radio bridge, Android/PC as cryptographic endpoints, Ollama as offline intelligence layer.

Paper title suggestion: "Design and Implementation of a Post-Quantum Cryptography Protected LoRa IoT Gateway Communication System with Integrated Edge AI"

# 8. Threat Model

## 8.1  Why Post-Quantum

Classical asymmetric cryptography (RSA, ECC, Diffie-Hellman) is vulnerable to Shor's algorithm running on a sufficiently powerful quantum computer. NIST standardised Kyber (ML-KEM) and Falcon (ML-DSA/ML-KEM) in 2024 specifically to replace these algorithms.

## 8.2  Grover's Algorithm and AES-256

Grover's algorithm provides a quadratic speedup for searching unsorted databases, which effectively halves the security level of symmetric keys against a quantum attacker. This means AES-128 (128-bit security classically) offers only ~64-bit security against a quantum computer. AES-256 retains ~128-bit effective security even under Grover's attack. This is why this system uses AES-256-GCM and not AES-128.

Note: Grover's algorithm is mentioned here as the threat that motivates AES-256. It is not implemented anywhere in the system — it requires a real quantum computer to run and has no place in an embedded or desktop application.

## 8.3  Attack Scenarios Addressed

| Attack | Mitigation in This System |
| --- | --- |
| Passive LoRa eavesdropping | All packets are AES-256-GCM encrypted. Without the session key, ciphertext is unreadable. |
| Man-in-the-middle during handshake | Falcon-512 signatures authenticate both endpoints. A fake public key is rejected. |
| Quantum computer breaking key exchange | ML-KEM (Kyber-768) is quantum-resistant by NIST standard. |
| Replay attacks | GCM nonce increments per packet. Replayed packets have wrong nonce and fail GCM tag verification. |
| Quantum brute force on AES | AES-256 retains 128-bit security under Grover's attack. Computationally infeasible. |
| Physical device compromise | Ephemeral session keys are discarded after each session. Old traffic cannot be decrypted. |
| Flash dump of ESP32 | ESP32 holds no keys. It only carries encrypted ciphertext. Dumping flash yields nothing. |

# 9. Sustainable Development Goals

| SDG | Contribution |
| --- | --- |
| SDG 9 — Industry, Innovation and Infrastructure | Develops resilient communication infrastructure for areas without internet. Integrates AI, IoT, LoRa, and PQC. |
| SDG 11 — Sustainable Cities and Communities | Provides emergency communication during disasters and network outages. Supports remote and underserved communities. |
| SDG 3 — Good Health and Well-being | Enables emergency communication in hospitals, ambulances, and rural healthcare facilities. |
| SDG 13 — Climate Action | Supports communication during climate-related disasters (floods, earthquakes, cyclones). |
| SDG 16 — Peace, Justice and Strong Institutions | PQC-secured communications for government, emergency response, and public safety. |

# 10. Implementation Phases

| Phase | Tasks | Deliverable |
| --- | --- | --- |
| Phase 1 Hardware Setup | Assemble ESP32 S3 + EBYTE E32 + antenna. Flash TinyUSB CDC firmware. Verify USB-C connection from Android and PC. | Working USB-C ↔ LoRa bridge. Raw byte pass-through confirmed. |
| Phase 2 LoRa Comms | Implement packet structure (type, seq, CRC16). Test range and throughput. Calibrate air data rate. | Two ESP32 nodes exchanging packetised data reliably. |
| Phase 3 PQC Handshake | Implement Kyber-768 + Falcon-512 on Android (Bouncy Castle) and PC (liboqs). Run handshake over LoRa link. | Shared AES-256 session key established between Android and PC over LoRa. |
| Phase 4 Mode 1 & 2 | Build Android app chat UI + AI query UI. Build PC FastAPI server + Ollama integration. Test end-to-end. | Text chat and AI text query working, encrypted, over LoRa. |
| Phase 5 Mode 3 & 4 | Integrate Codec2 JNI on Android. Implement audio packetisation and reassembly. Integrate Android STT for Mode 4. | Voice messages and voice-to-AI working end-to-end. |
| Phase 6 Evaluation | Measure: range, latency per mode, packet loss, handshake time, power consumption. Document results. | Evaluation data for paper. System demo ready. |

# 11. Evaluation Metrics

| Metric | Measurement Method |
| --- | --- |
| Communication range | Measure maximum distance at which packet loss < 5% for each LoRa air data rate setting. |
| Message latency (Mode 1) | Timestamp at Android A send and Android B receive. Average over 50 messages. |
| AI response latency (Mode 2 & 4) | Time from query send to full response displayed. Average over 20 queries. |
| Audio transfer time (Mode 3) | Time for a 10-second voice message to fully play on receiver. Average over 10 recordings. |
| Packet delivery rate | Count delivered vs sent packets over 100 transmissions at 50m, 500m, 1km, 5km. |
| Handshake time | Measure ML-KEM + Falcon handshake duration. Average over 20 sessions. |
| Security performance | Demonstrate that a third ESP32 sniffing LoRa channel cannot read any plaintext. |
| Power consumption | Current draw of ESP32 + EBYTE E32 during transmit, receive, and idle. Measure with multimeter. |

Document prepared for: Abdul Rahman